SAMPLE TECHNICAL TRAINING INSTITUTE

P.O. Box 000-00100 · NAIROBI

assessment@sample-tti.ac.ke

OFFICE OF EXAMINATION & ASSESSMENT OFFICER

ICT TECHNICIAN  LEVEL 6

0612 351 03A  ·  0612 351 03A/W1  ·  ICT/OS/CN/CR/03/6

COMPUTER NETWORK SETUP

SERIES 1 · 2026

OBSERVATION CHECKLIST 1

| Candidate's Name & Registration Code |  |  |  |  |
| --- | --- | --- | --- | --- |
| Assessor's Name & Registration Code |  |  |  |  |
| Venue of Assessment |  |  |  |  |
| Date of Assessment |  |  |  |  |
| Items to be Evaluated: Please award marks as appropriate, tick the competence reached and give a brief comment on your observation. | Marks Available | Marks Obtained | Competent / Not Yet Competent | Comments |
| TASK 1: Terminate and test patch cords: |  |  |  |  |
| [PC 1.5] Selected the correct termination tools and confirmed each was serviceable before starting. Marking points: Crimper, stripper, cutters and tester all selected; any defective tool identified and set aside. | 5 |  | C ☐ NYC ☐ |  |
| [PC 1.7] Stripped the outer sheath to the manufacturer's specified length without nicking the conductors. Marking points: Strip length 12–13 mm on every connector; no visible nick or copper showing through insulation. | 7 |  | C ☐ NYC ☐ |  |
| [PC 1.8] Untwisted and arranged the conductors in the correct T568A/T568B sequence. Marking points: Untwist under 13 mm; colour sequence correct for the declared scheme on every end. | 8 |  | C ☐ NYC ☐ |  |
| [PC 1.9] Crimped RJ-45 connectors so all eight conductors seat fully and the strain relief captures the sheath. Marking points: All eight conductors visible at the connector tip; boot fitted; sheath held by the strain relief; connector does not pull off under hand tension. | 8 |  | C ☐ NYC ☐ |  |
| [PC 1.10] Produced one correct crossover and two correct straight-through cords as specified. Marking points: Crossover shows the 1-3 / 2-6 transposition on the tester; both straight-through cords map 1-to-1. | 4 |  | C ☐ NYC ☐ |  |
| Subtotal 1 | 32 |  |  |  |
| TASK 2: Terminate an outlet to a patch panel and prove the link: |  |  |  |  |
| [PC 1.4] Identified the correct cable type for the horizontal run and cut it to length with a service loop. Marking points: Cat 6 UTP selected; service loop left at both ends; no kinks or bend radius violations. | 4 |  | C ☐ NYC ☐ |  |
| [PC 1.6] Punched the conductors down onto the outlet and the patch panel to the chosen scheme. Marking points: Same scheme both ends; conductors fully seated; excess trimmed flush by the tool. | 6 |  | C ☐ NYC ☐ |  |
| [PC 1.3] Selected and correctly connected the devices needed to demonstrate the link. Marking points: Patch cord, switch port and laptop connected correctly; link light observed and shown to the assessor. | 5 |  | C ☐ NYC ☐ |  |
| [PC 1.1] Identified the topology the completed link forms and explained it to the assessor. Marking points: Star topology named, with the switch identified as the central node. | 3 |  | C ☐ NYC ☐ |  |
| [PC 1.2] Named the network components used in the completed link. Marking points: NIC, cable, outlet, patch panel and switch all correctly named. | 3 |  | C ☐ NYC ☐ |  |
| [PC 1.11] Tested the completed link for continuity and pinout and recorded the result. Marking points: Wire map read on the tester; pass/fail entered on the test schedule against the run label. | 5 |  | C ☐ NYC ☐ |  |
| [PC 1.12] Cleared the work area, disposed of waste and returned all tools. Marking points: Offcuts cleared, bench wiped, every tool signed back into the store. | 2 |  | C ☐ NYC ☐ |  |
| Subtotal 2 | 28 |  |  |  |
| GRAND TOTAL | 60 |  |  |  |

ASSESSMENT OUTCOME

The candidate was found to be:    Competent  ☐    Not Yet Competent  ☐

(The candidate is competent if they obtain at least 50 % overall AND are judged Competent on every performance criterion above.)

Feedback from the Candidate: ____________________________________________________________

Feedback to the Candidate:   ____________________________________________________________

Candidate's Signature: ______________________     Date: ____________

Assessor's Signature:  ______________________     Date: ____________