TOOLS

0612 351 03A  ·  END OF MODULE · 2026

COMPUTER NETWORK SETUP

SAMPLE TECHNICAL TRAINING INSTITUTE

P.O. Box 000-00100 · NAIROBI

assessment@sample-tti.ac.ke

DEPARTMENT:  INFORMATION COMMUNICATION TECHNOLOGY

SUMMATIVE OBSERVATION CHECKLIST — VARIANT A

| Candidate's Name & Registration Code |  |  |  |  |
| --- | --- | --- | --- | --- |
| Assessor's Name & Registration Code |  |  |  |  |
| Venue of Assessment |  |  |  |  |
| Date of Assessment |  |  |  |  |
| Items to be Evaluated: Please award marks as appropriate, tick the competence reached and give a brief comment on your observation. | Marks Available | Marks Obtained | Competent / Not Yet Competent | Comments |
| TASK 1: Terminate and certify the horizontal cabling: |  |  |  |  |
| [PC 1.5] Selected serviceable tools and the correct cable and connectors for the specification. Marking points: Crimper, stripper, punch-down tool and tester selected and checked; Cat 6 cable and Cat 6 connectors used. | 5 |  | C ☐ NYC ☐ |  |
| [PC 2.4] Routed and dressed the runs within bend radius and trunking fill limits. Marking points: No kink or sharp bend; trunking not overfilled; service loop at each end; data separated from power. | 8 |  | C ☐ NYC ☐ |  |
| [PC 1.7] Stripped sheath to the specified length without damaging the conductors. Marking points: 12–13 mm strip on every termination; no nicked insulation. | 6 |  | C ☐ NYC ☐ |  |
| [PC 1.8] Arranged conductors in the declared scheme with untwist within limits. Marking points: T568B at every point; untwist under 13 mm; sequence correct on all six terminations. | 8 |  | C ☐ NYC ☐ |  |
| [PC 2.3] Punched down the outlets and patch panel ports correctly. Marking points: Conductors fully seated by the tool; excess trimmed flush; no conductor left proud of the IDC. | 6 |  | C ☐ NYC ☐ |  |
| [PC 1.11] Certified or tested every run and cord and recorded the results. Marking points: Wire map read and recorded for all runs and cords; failures re-terminated and re-tested; schedule complete and legible. | 7 |  | C ☐ NYC ☐ |  |
| Subtotal 1 | 40 |  |  |  |
| TASK 2: Connect and configure the network: |  |  |  |  |
| [PC 2.2] Mounted the devices in the cabinet with correct positioning and airflow. Marking points: Panel above or below its switch; fixings secure; airflow unobstructed; power leads clear of data cords. | 6 |  | C ☐ NYC ☐ |  |
| [PC 2.5] Interconnected the devices to form the specified topology. Marking points: Workstations to switch, switch uplink to router; star topology correctly formed and explained. | 8 |  | C ☐ NYC ☐ |  |
| [PC 2.6] Produced and applied a correct IP addressing scheme. Marking points: Addresses within 192.168.50.0/24, no duplicates, correct mask and gateway on every device; plan documented before configuring. | 8 |  | C ☐ NYC ☐ |  |
| [PC 2.1] Observed safety measures and PPE throughout the task. Marking points: PPE worn; power isolated before cabinet work; no trip hazards left in the walkway. | 5 |  | C ☐ NYC ☐ |  |
| [PC 2.7] Verified connectivity across the network and recorded each result. Marking points: Ping to peer, switch, gateway and external host all demonstrated and recorded; link speed confirmed. | 8 |  | C ☐ NYC ☐ |  |
| Subtotal 2 | 35 |  |  |  |
| TASK 3: Diagnose, repair and document a fault: |  |  |  |  |
| [PC 3.1] Selected appropriate monitoring or test tools for the symptom presented. Marking points: Tool choice matches the layer being investigated — tester for cabling, ipconfig/ping for addressing, switch statistics for port errors. | 4 |  | C ☐ NYC ☐ |  |
| [PC 3.2] Monitored the network and interpreted the indicators correctly. Marking points: Link lights, port statistics or ping results read and correctly interpreted to the assessor. | 5 |  | C ☐ NYC ☐ |  |
| [PC 3.3] Followed a structured diagnostic procedure and localised the fault. Marking points: Worked outward from the symptom in a logical order; substitution used to isolate; the fault correctly identified with supporting evidence. | 7 |  | C ☐ NYC ☐ |  |
| [PC 3.4] Repaired or replaced the faulty component and proved the network restored. Marking points: Repair carried out safely and correctly; connectivity re-tested and demonstrated to the assessor. | 6 |  | C ☐ NYC ☐ |  |
| [PC 3.5] Completed a maintenance log entry to workplace standard. Marking points: Date, fault reported, fault found, action taken, verification result and technician name all recorded. | 3 |  | C ☐ NYC ☐ |  |
| Subtotal 3 | 25 |  |  |  |
| GRAND TOTAL | 100 |  |  |  |

ASSESSMENT OUTCOME

The candidate was found to be:    Competent  ☐    Not Yet Competent  ☐

(The candidate is competent if they obtain at least 50 % overall AND are judged Competent on every performance criterion above.)

Feedback from the Candidate: ____________________________________________________________

Feedback to the Candidate:   ____________________________________________________________

Candidate's Signature: ______________________     Date: ____________

Assessor's Signature:  ______________________     Date: ____________